| RADA SMĚRNICE RADY z 22. března 1982 o mezních hodnotách a jakostních cílech při vypouštění rtuti z elektrolytické výroby chloru a alkalických hydroxidů (ve znění dodatků) 1) 82/176/EHS (OJ L 81, 27.3.1982, s.29) RADA EVROPSKÝCH SPOLEČENSTVÍ, se zřetelem na Smlouvu ustavující Evropské hospodářské společenství a zvláště na její články 100 a 235, se zřetelem na Směrnici Rady 76/464/EHS 2) ze 4. května 1976 o znečištění způsobeném určitými nebezpečnými látkami, vypouštěnými do vodního prostředí Společenství , a zvláště na její čl. 6, se zřetelem na návrh Komise 3), se zřetelem na stanovisko Evropského parlamentu 4), se zřetelem na stanovisko Hospodářského a sociálního výboru 5), vzhledem k tomu, že pro účely ochrany vodního prostředí Společenství před znečištěním způsobeném určitými nebezpečnými látkami, zavádí článek 3 Směrnice 76/464/EHS systém předchozího povolení, které stanoví emisní standardy pro vypouštění látek v Seznamu I její Přílohy; 1) Změny dle Směrnice Rady 90/656/EHS z 4. prosince 1990 o přechodných opatřeních uplatňovaných v Německu v souvislosti s určitými předpisy Společenství na ochranu životního prostředí (OJ L 353, 17.12.1990, s.59) a dle Směrnice Rady 91/692/EHS z 23.prosince ke standardizaci a racionalizaci zpráv o zavádění určitých Směrnic vztahujících se k životnímu prostředí (OJ L 377, 31.12.1991, s.48). 2) OJ L 129, 18.5.1976, s.23 3) OJ C 169, 6.7.1979, s.2 4) OJ C 341, 31.12.1980, s.24 5) OJ C 83, 2.4.1980, s.16 vzhledem k tomu, že článek 6 stejné Směrnice zajišťuje, že musí být stanoveny mezní hodnoty pro takovéto emisní standardy a rovněž jakostní cíle pro vodní prostředí ovlivněné vypouštěním těchto látek; vzhledem k tomu, že rtuť a její sloučeniny jsou uvedeny v Seznamu I; vzhledem k tomu, že na Členských státech se požaduje, aby uplatňovaly mezní hodnoty, s výjimkou případů, kde mohou použít jakostních cílů; vzhledem k tomu, že jelikož znečištění způsobené vypouštěním rtuti do vod je ve značné míře způsobováno průmyslem elektrolýzy alkalických chloridů, měly by být nejprve stanoveny mezní hodnoty pro tento průmysl a určeny jakostní cíle pro vodní prostředí, do něhož je rtuť tímto průmyslem vypouštěna; vzhledem k tomu, že takováto vypouštění by proto měla podléhat předchozímu povolení; vzhledem k tomu, že účelem takovýchto jakostních cílů musí být eliminace znečištění různých částí vodního prostředí rtutí, které by mohlo být ovlivněno vypouštěním z průmyslu chlor-alkalické elektrolýzy, obsahujícím rtuť; vzhledem k tomu, že takovéto jakostní cíle musí být stanoveny výslovně k tomuto účelu a nikoliv s úmyslem stanovit pravidla týkající se ochrany spotřebitele nebo trhu výrobků z vodního prostředí; vzhledem k tomu, že by se měl být zavést specifický monitorovací postup, umožňující prokázat Členským státům, že jakostní cíle jsou dodržovány; vzhledem k tomu, že by měla být učiněna opatření, aby Členské státy monitorovaly vodní prostředí, ovlivněné výše uvedeným vypouštěním rtuti, pro účely účinného uplatňování této Směrnice; vzhledem k tomu, že článek 6 Směrnice 76/464/EHS neobsahuje pravomoci pro zavedení takovéhoto monitorování; vzhledem k tomu, že jelikož potřebné pravomoci nejsou uvedeny ve Smlouvě, oporou by měl být její článek 235; vzhledem k tomu, že je důležité, aby Komise předávala Radě každých pět let srovnávací hodnocení o zavádění této Směrnice Členskými státy; vzhledem k tomu, že jelikož podzemní vody jsou předmětem zvláštní Směrnice, jsou vyňaty z působnosti této Směrnice, PŘIJALA TUTO SMĚRNICI: Článek 1 - Tato Směrnice: - stanoví, podle článku 6(1) Směrnice 76/464/EHS, mezní hodnoty emisních standardů pro rtuť při vypouštění z průmyslových závodů, jak definuje článek 2(d) této Směrnice, - stanoví, podle článku 6(2) Směrnice 76/464/EHS, jakostní cíle pro rtuť ve vodním prostředí, - stanoví, podle článku 6(4) Směrnice 76/464/EHS, lhůty ke splnění podmínek povolení, vydaných příslušnými úřady Členských států, v případech existujících vypouštění, - stanoví podle článku 12(1) Směrnice 76/464/EHS, referenční měřicí metody, umožňující určit obsah rtuti při vypouštění a ve vodním prostředí, - ustanovuje, podle článku 6(3) Směrnice 76/464/EHS, monitorovací postup, - žádá Členské státy, aby navzájem spolupracovaly v případě vypouštění, které ovlivňuje vody více než jednoho Členského státu. - Tato Směrnice platí pro vody uvedené v článku 1 Směrnice 76/464/EHS, s výjimkou podzemních vod. Článek 2 Pro účely této Směrnice: - „rtuť“ je: - chemický prvek rtuť, - rtuť obsažená v jakékoliv její sloučenině; - „mezní hodnoty“ jsou: hodnoty specifikované v Příloze I; - „jakostní cíle“ jsou: požadavky specifikované v Příloze II; - „průmyslový závod“ je: závod používající chlor-alkalické elektrolýzy ve rtuťových vanách, - „existující závod“ je: průmyslový závod, který je v provozu k datu notifikace této Směrnice; - „nový závod“ je: - průmyslový závod, který zahájí provoz po datu notifikace této Směrnice, - existující průmyslový závod, jehož kapacita elektrolýzy alkalických chloridů ve rtuťových vanách se podstatně zvýší po datu notifikace této Směrnice. Článek 3 - Mezní hodnoty, lhůty, ve kterých musí být tyto dodrženy a monitorovací postup pro vypouštění, jsou stanoveny v Příloze I. - Povolení podle článku 3 Směrnice 76/464/EHS musí obsahovat opatření nejméně tak přísná jako ta v Příloze I této Směrnice s výjimkou, kde Členský stát dodržuje článek 6(3) Směrnice 76/464/EHS na základě Příloh II a IV této Směrnice. Povolení se mají znovu posuzovat nejméně každé čtyři roky. - Bez újmy ke svým povinnostem, vyplývajících z odstavců 1 a 2, a k ustanovením Směrnice 76/464/EHS, mohou Členské státy vydat povolení novým závodům jen tehdy, jestliže tyto uplatní standardy odpovídající nejlepším dostupným technickým prostředkům pro prevenci vypouštění rtuti. Pokud Členský stát přijme jakoukoliv metoda, kde pro technické důvody uvažovaná opatření neodpovídají nejlepším dostupným technickým prostředkům, poskytne Komisi, před vydáním jakéhokoliv povolení, důkazy obhajující tyto důvody. Komise zašle ve lhůtě tří měsíců Členským státům zprávu, oznamující její stanovisko ke zmírnění ve smyslu druhého pododstavce. - Referenční metoda rozboru používaná ke stanovení přítomnosti rtuti je dána v Příloze III(1). Jiné metody mohou být použity za předpokladu, že meze detekce, přesnost a správnost takovýchto metod jsou přinejmenším takové, jako u těch, které jsou stanoveny v Příloze III(1). Přesnost požadovaná pro měření průtoku při vypouštění, je dána v Příloze III(2). Článek 4 Dotčené Členské státy odpovídají za monitorování vodního prostředí, které je ovlivněno průmyslovým vypouštěním. V případě vypouštění, které ovlivňuje vody více Členských států, musí dotčené Členské státy spolupracovat tak, aby harmonizovaly monitorovací postupy. Článek 5 - Členské státy zašlou Komisi každé tři roky informace o zavádění této Směrnice formou dílčí zprávy, která bude přiměřeně zahrnovat i další související směrnice Společenství,. Tato zpráva bude zpracována na základě formulářů a dotazníků navržených Komisí na základě postupu dle článku 6 Směrnice 91/692/EHS 6). Dotazníky nebo formuláře se zašlou členským státům šest měsíců před počátkem období, které tato zpráva zahrnuje. Zpráva se odešle Komisi do devíti měsíců od konce tříletého období, které zahrnuje. První zpráva bude pokrývat období od r. 1993 do r. 1995 včetně. Komise publikuje zprávu o zavádění Směrnice za Společenství do devíti měsíců od obdržení zpráv od Členských států. - V případě změny ve vědeckém poznání, zásadně se týkajícího toxicity, persistence a akumulace rtuti v živých organismech a sedimentech nebo v případě zlepšení nejlepších dostupných technických prostředků, Komise předloží Radě, pokud je to třeba, příslušné návrhy s cílem zpřísnit mezní hodnoty a jakostní cíle. Článek 6 - Členské státy zavedou opatření potřebná k přizpůsobení se této Směrnici před 1. červencem 1983. O tom musí bez odkladu informovat Komisi. - Členské státy sdělí Komisi texty opatření v národních zákonech, které přijaly v oblasti pojící se s touto Směrnicí. Článek 7 Tato Směrnice je určena členským státům. Brusel, 22. března 1982. za Radu: L. Tindemans, president 6) OJ L 377, 31.12.1991, s.48 Příloha I Mezní hodnoty, lhůty ve kterých musí být splněny a monitorovací postup pro vypouštění - Mezní hodnoty vyjádřené v koncentracích, které zásadně nesmějí být překročeny, jsou v následující tabulce: |  |  |  |  |  |  |  |  | COUNCIL COUNCIL DIRECTIVE of 22 March 1982 on limit values and quality objectives for mercury discharges by the chlor-alkali electrolysis industry (consolidated version) 1) 82/176/EEC (OJ L 81, 27.3.1982, p.29) THE COUNCIL OF THE EUROPEAN COMMUNITIES, Having regard to the Treaty establishing the European Economic Community, and in particular Articles 100 and 235 thereof, Having regard to Directive 76/464/EEC 2) of 4 May 1976 on pollution caused by certain dangerous substances discharged into the aquatic environment of the Community, and in particular Article 6 thereof, Having regard to the proposal from the Commission 3), Having regard to the opinion of the European Parliament 4), Having regard to the opinion of the Economic and Social Committee 5), Whereas, in order to protect the aquatic environment of the Community against pollution by certain dangerous substances, Article 3 of Directive 76/464/EEC provides for a system of prior authorization laying down emission standards for discharges of the substances in List I in the Annex thereto; 1) Amendments according. to the Council Directive 90/656/EEC of 4 December 1990 on the transitional measures applicable in Germany with regard to certain Community provisions relating to the protection of the environment (OJ L 353, 17.12.1990, p.59) and Council Directive 91/692/EEC of 23 December 1991 standardizing and rationalizing reports on the implementation of certain Directives relating to the environment (OJ L 377, 31.12.1991, p.48). 2) OJ L 129, 18.5.1976, p.23 3) OJ C 169, 6.7.1979, p.2 4) OJ C 341, 31.12.1980, p.24 5) OJ C 83, 2.4.1980, p.16 Whereas Article 6 of the same Directive provides that limit values shall be laid down for such emission standards and also quality objectives for the aquatic environment affected by these substances; Whereas mercury and its compounds are included in List I; Whereas the Member States are required to apply the limit values except in the cases where they may employ quality objectives; Whereas, since the pollution caused by discharges of mercury into water is caused, to a large extent, by the electrolysis of alkali chlorides, in the first instance limit values should be established for this industry and quality objectives should be laid down for the aquatic environment into which mercury is discharged by this industry; whereas such discharges should therefore require prior authorization; Whereas the purpose of such quality objectives must be to eliminate mercury pollution of the various parts of the aquatic environment which might be affected by mercury-bearing discharges from the chlor-alkali electrolysis industry; Whereas such quality objectives must be laid down expressly for this purpose and not with the intention of establishing rules pertaining to consumer protection or to the marketing of products from the aquatic environment; Whereas a specific monitoring procedure should be instituted to enable the Member States to demonstrate that the quality objectives are complied with; Whereas provision should be made for the monitoring by the Member States of the aquatic environment affected by the said mercury discharges with a view to efficient application of this Directive; whereas Article 6 of Directive 76/464/EEC does not provide for the powers to introduce such monitoring; whereas, since the necessary powers of action have not been provided for in the Treaty, Article 235 thereof should be invoked; Whereas it is important that the Commission forward to the Council, every five years, a comparative assessment of the implementation of this Directive by the Member States; Whereas, since groundwater is the subject of a specific Directive, it is excluded from the scope of this Directive, HAS ADOPTED THIS DIRECTIVE: Article 1 - This Directive: - in pursuance of Article 6(1) of Directive 76/464/EEC, lays down limit values for emission standards for mercury in discharges from industrial plants as defined in Article 2(d) of this Directive, - in pursuance of Article 6(2) of Directive 76/464/EEC, lays down quality objectives for mercury in the aquatic environment, - in pursuance of Article 6(4) of Directive 76/464/EEC, lays down the time limits for compliance with the conditions of the authorizations granted by the competent authorities of Member States in the case of existing discharges, - in pursuance of Article 12(1) of Directive 76/464/EEC, lays down the reference methods of measurement enabling the mercury content in discharges and in the aquatic environment to be determined, - in pursuance of Article 6(3) of Directive 76/464/EEC, establishes a monitoring procedure, - requires Member States to cooperate with one another in the case of discharges affecting the waters of more than one Member State. - This Directive applies to the waters referred to in Article 1 of Directive 76/464/EEC with the exception of groundwater. Article 2 For the purposes of this Directive: - 'mercury' means: - the chemical element mercury, - the mercury contained in any of its compounds; - 'limit values' means: the values specified in Annex I; - 'quality objectives' means: the requirements specified in Annex II; - 'industrial plant' means: a plant in which alkali chlorides are electrolyzed by means of mercury cells; - 'existing plant' means: an industrial plant which is operational on the date of notification of this Directive; - 'new plant' means: - an industrial plant which has become operational after the date of notification of this Directive, - an existing industrial plant whose capacity for the electrolysis of alkali chlorides by means of mercury cells has been substantially increased after the date of notification of this Directive. Article 3 - The limit values, the time limits by which they must be complied with and the monitoring procedure for discharges are laid down in Annex I. - The authorizations referred to in Article 3 of Directive 76/464/EEC must contain provisions at least as stringent as those in Annex I to this Directive, except in cases where a Member State is complying with Article 6(3) of Directive 76/464/EEC on the basis of Annexes II and IV to the present Directive. The authorizations shall be reviewed at least every four years. - Without prejudice to their obligations arising out of paragraphs 1 and 2 and the provisions of Directive 76/464/EEC, Member States may grant authorizations for new plants only if such authorizations contain a reference to the standards corresponding to the best technical means available for preventing discharges of mercury. Whatever the method it adopts, the Member State, where for technical reasons the intended measures do not conform to the best technical means available, shall provide the Commission, before any authorization, with the justifications for these reasons. Within three months, the Commission shall send a report to the Member States stating its opinion on the derogation covered by the second subparagraph. - The reference method of analysis for determining the presence of mercury are given in Annex III(1). Other methods may be used provided that the limits of detection, precision and accuracy of such methods are at least as good as those laid down in Annex III(1). The accuracy required in the measurement of effluent flow is given in Annex III(2). Article 4 The Member States concerned shall be responsible for monitoring the aquatic environment affected by industrial discharges. In the case of discharges affecting the waters of several Member States, the Member States concerned shall cooperate with a view to harmonizing monitoring procedures. Article 5 - At intervals of three years the Member States shall send information to the Commission on the implementation of this Directive, in the form of a sectoral report which shall also cover other pertinent Community Directives. This report shall be drawn up on the basis of a questionnaire or outline drafted by the Commission in accordance with the procedure laid down in Article 6 of Directive 91/692/EEC 6) The questionnaire or outline shall be sent to the Member States six months before the start of the period covered by the report. The report shall be sent to the Commission within nine months of the end of the three-year period covered by it. The first report shall cover the period from 1993 to 1995 inclusive. The Commission shall publish a Community report on the implementation of the Directive within nine months of receiving the reports from the Member States. In the event of a change in scientific knowledge relating principally to the toxicity, persistence and accumulation of mercury in living organisms and sediments or in the event of an improvement in the best technical means available, the Commission shall submit appropriate proposals to the Council with the aim of reinforcing, if necessary, the limit values and the quality objectives. Article 6 - Member States shall bring into force the measures necessary to comply with this Directive before 1 July 1983. They shall forthwith inform the Commission thereof. - Member States shall communicate to the Commission the text of the provisions of national law which they adopt in the field governed by this Directive. Article 7 This Directive is addressed to the Member States. Done at Brussels, 22 March 1982. For the Council: The president L. Tindemans, 6) OJ L 377, 31.12.1991, p.48 ANNEX I Limit values, time limits by which they must be complied with, and monitoring procedure for discharges - The limit values expressed in terms of concentration which, in principle, should not be exceeded are set out in the following table: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Předmět měření | Průměrné měsíční mezní hodnoty platné od: |  |  |  |  |  |  | Unit of measurement |  |  | Monthly average limit values not to be exceeded from 1July |  |  |  |
|  | 1.7.1983 |  | 1.7.1986 |  |  |  |  |  |  |  | 1983 |  | 1986 |  |
| Recyklovaná a ztracená solanka 1 g rtuti na litr | 75 |  | 50 |  |  |  |  | Recycled brine and lost brine 1 Micrograms of mercury per litre |  |  | 75 |  | 50 |  |
| 1 Platné pro celkové množství rtuti ve všech rtuť obsahujících vodách vypouštěných ze závodu. Ve všech případech, mezní hodnoty vyjádřené jako maximální koncentrace nesmějí být vyšší než ty, které jsou vyjádřeny jako maximální množství dělená spotřebovaným množstvím vody na tunu instalované výrobní kapacity. - Poněvadž však koncentrace rtuti ve výpustích závisí na objemu obsažené vody, který je pro různé procesy a závody odlišný, mezní hodnoty vyjádřené množstvím vypouštěné rtuti ve vztahu k instalované výrobní kapacitě chloru a udané v následující tabulce, musejí být ve všech případech respektovány. |  |  |  |  |  |  |  |  | 1.Applicable to the total quantity of mercury present in all mercury-containing water discharged from the site of the industrial plant In all cases, limit values expressed as maximum concentrations may not be greater than those expressed as maximum quantities divided by water requirements per tonne of installed chlorine production capacity. - However, because the concentration of mercury in effluents depends upon the volume of water involved, which is different for different processes and plants, the limit values expressed in terms of quantity of mercury discharged in relation to installed chlorine production capacity given in the following table must be observed in all cases. |  |  |  |  |  |
| - Předmět měření |  | Průměrné měsíční mezní hodnoty platné od 1.června |  |  |  |  |  |  |  | Unit of measurement |  | Monthly average limit values not to be exceeded from 1 July |  |  |
|  |  | 1983 |  | 1986 |  |  |  |  |  |  |  | 1983 |  | 1986 |
| Recyklovaná solanka 1 gramy rtuti na tunu instalované výrobní kapacity chloru |  | 0,5 |  | 0,5 |  |  |  |  |  | Recycled brine 1 Grams of mercury per tonne of installed chlorine production capacity |  | 0.5 |  | 0.5 |
| 2 |  | 1,5 |  | 1,0 |  |  |  |  |  | 2 |  | 1,5 |  | 1.0 |
| Ztracená solanka 2 gramy rtuti na tunu instalované výrobní kapacity chloru |  | 8,0 |  | 5,0 |  |  |  |  |  | Lost brine1 Grams of mercury per tonne of installed chlorine production capacity |  | 8.0 |  | 5.0 |
| 1 Hodnoty platné pro rtuť obsaženou ve výpustích z výrobní jednotky chloru. 2 Hodnoty platné pro celkové množství rtuti ve všech rtuť obsahujících vodách vypouštěných ze závodu. - Denní průměrné mezní hodnoty jsou čtyřnásobkem odpovídající měsíční průměrné hodnoty udané v bodech 1 a 2. - Ke kontrole výpustí, zda splňují emisní normy stanovené v souladu s mezními hodnotami danými v této Příloze, musí být zaveden monitorovací postup. Tento postup musí obsahovat: - každodenní odběr reprezentativních vzorků z výpustí po celých 24 hodin a měření koncentrací rtuti v těchto vzorcích, a - měření celkového průtoku ve výpustích v tomto období. Množství rtuti vypouštěné za měsíc se počítá jako součet denních množství v daném měsíci. Tento součet se pak dělí instalovanou výrobní kapacitou chloru. Příloha II Jakostní cíle U těch Členských států, které využívají výjimky citované v čl. 6(3) Směrnice 76/464/EHS, budou emisní normy, které Členské státy musejí určit a zabezpečit jejich aplikaci, stanoveny tak, aby přiměřené jakostní cíle (viz níže) byly dodržovány v oblasti zasažené výpusťmi rtuti z průmyslu elektrolýzy alkalických chloridů. Kompetentní úřad určí postiženou oblast v každém z případů a vybere z jakostních cílů uvedených v článku 1 takové jakostní cíle, o nichž soudí, že jsou vhodné k použití v postižené oblasti, a že přispějí k účelu této Směrnice, totiž k zabránění znečišťování. - Následující jakostní cíle jsou stanoveny v souladu s čl. 2 Směrnice 76/464/EHS s cílem eliminovat znečišťování ve smyslu této Směrnice. - Koncentrace rtuti v reprezentativním vzorku rybího masa zvoleného jako indikátor, nesmí přestoupit hodnotu 0,3 mg/kg vlhkého masa. - Celková koncentrace rtuti ve vnitrozemských povrchových vodách postižených výpusťmi nesmí překročit hodnotu 1 g/l jako aritmetický průměr výsledků během roku. - Celková koncentrace rtuti v ústích řek postižených výpusťmi nesmí překročit hodnotu 0,5 g/l jako aritmetický průměr výsledků během roku. - Celková koncentrace rtuti v teritoriálních vodách a vnitřních pobřežních vodách jiných než ústí řek postižených výpusťmi nesmí překročit hodnotu 0,3 g/l jako aritmetický průměr výsledků během roku. - Kvalita vody musí být dostatečná, aby splnila požadavky každé jiné Směrnice Rady platné u těchto vod, pokud se týká přítomnosti rtuti. - Koncentrace rtuti v sedimentech nebo korýších nesmí časově podstatně vzrůstat. - Jestliže je u vod v oblasti použito několika jakostních cílů, kvalita vody musí být dostatečná, aby splňovala každý z těchto cílů. - Číselné hodnoty jakostních cílů specifikovaných v odstavcích 1.2, 1.3 a 1.4 mohou být výjimečně a tam kde je to nutné z technických důvodů, zvýšeny 1,5x až do 30. června 1986 za předpokladu, že Komise bude předem informována. Příloha IIi Referenční měřicí metody - Referenční analytická metoda pro stanovení rtuti vodách, rybím mase, sedimentech a korýších je bezplamenná atomová adsorpční spektrofotometrie po vhodné úpravě vzorku, která především respektuje prvotní oxidaci rtuti a postupnou redukci rtuťnatých iontů. Mez stanovení 1 musí být taková aby koncentrace rtuti byla měřena se správností 1 a přesností 1 +/- 30% při následujících koncentracích: - v případě výpustí, desetina z dovolené maximální koncentrace rtuti udané v oprávnění; - u vnitrozemských povrchových vod, desetina koncentrace rtuti udané jako jakostní cíl, - v případě rybího masa nebo korýšů, desetina koncentrace rtuti udané jako jakostní cíl, - v případě sedimentů, desetina koncentrace rtuti ve vzorku nebo 0,05 mg/kg sušiny, je-li tato hodnota vyšší. - Průtok se měří se správností +/- 20%. Definice těchto termínů jsou ve Směrnici Rady 79/869/EHS z 9. října 1979 o metodách měření, četnosti odběrů a rozborů povrchových vod určených k odběru pitné vody v Členských státech (OJ L 271, 29.10.1979, s.44). Příloha IV Monitorovací postup pro jakostní cíle - U každého oprávnění uděleného ve shodě s touto Směrnicí, určí kompetentní úřad podrobné předpisy , monitorovací postupy a časové lhůty k dodržování příslušných jakostních cílů. - V Souladu s čl. 6(3) Směrnice 76/464/EHS oznámí Členský stát Komisi u každého zvoleného a použitého jakostního cíle: - místa výpustí a prostředky rozptylu, - oblast, v níž se používá jakostního cíle, - polohu míst odběru vzorků, - četnost odběru vzorků - metody odběru vzorků a měření, - získané výsledky. - Vzorky musejí být dostatečně reprezentativní co se týče kvality vodního prostředí v postižené oblasti a četnost odběrů vzorků musí být postačující k průkazu jakýchkoliv změn vodního prostředí včetně, a to zvláště, změn v hydrologickém režimu. Analýza ryb ze slaných vod musí být provedena na dostatečně reprezentativním počtu kusů a druhů. - S ohledem na jakostní cíl v části 1.1 Přílohy II, vybere kompetentní úřad druhy ryb, které budou analyzovány. Ve slaných vodách to jsou druhy, které žijí v pobřežních vodách a jsou místně loveny jako treska, bílá treska, platýz obecný (platessa vulgaris), makrela, treska skvrnitá a platejs (pleuronectes flesus). Prohlášení k čl. 3(3) Rada a Komise konstatují, že použití nejlepších dostupných technických prostředků umožňuje omezit výpusti rtuti u nově budovaných průmyslových závodů při aplikaci procesu s recyklovanou solankou na méně než 0,5 g/t instalované výrobní kapacity chloru. |  |  |  |  |  |  |  |  | 1 Applicable to the mercury present in effluent discharged from the chlorine production unit. 2 Applicable to the total quantity of mercury present in all mercury-containing water discharged from the site of the industrial plant. - The daily average limit values are four times the corresponding monthly average limit values given in points 1 and 2. - In order to check whether the discharges comply with the emission standards which have been fixed in accordance with the limit values laid down in this Annex, a monitoring procedure must be instituted. This procedure must provide for: - the taking each day of a sample representative of the discharge over a period of 24 hours and the measurement of the mercury concentration of that sample, and - the measurement of the total flow of the discharge over that period. The quantity of mercury discharged during a month must be calculated by adding together the quantities of mercury discharged each day during that month. This total must then be divided by the installed chlorine production capacity. ANNEX II Quality objectives For those Member States which apply the exception provided for in Article 6(3) of Directive 76/464/EEC, the emission standards which Member States must establish and ensure are applied, pursuant to Article 5 of that Directive, shall be fixed so that the appropriate quality objective or objectives from among those listed below is or are complied with in the area affected by discharges of mercury from the chlor-alkali electrolysis industry. The competent authority shall determine the area affected in each case and shall select from among the quality objectives listed in paragraph 1 the objective or objectives that it deems appropriate having regard to the intended use of the area affected, taking account of the fact that the purpose of this Directive is to eliminate all pollution. - In order to eliminate pollution as defined in Directive 76/464/EEC, and pursuant to Article 2 of that Directive, the following quality objectives are set: - The concentration of mercury in a representative sample of fish flesh chosen as an indicator must not exceed 0.3 mg/kg wet flesh. - The total concentration of mercury in inland surface waters affected by discharges must not exceed 1 g/l as the arithmetic mean of the results obtained over a year. - The concentration of mercury in solution in estuary waters affected by discharges must not exceed 0.5 g/l as the arithmetic mean of the results obtained over a year. - The concentration of mercury in solution in territorial sea waters and internal coastal waters other than estuary waters affected by discharges must not exceed 0.3 g/l as the arithmetic mean of the results obtained over a year. - The quality of the waters must be sufficient to comply with the requirements of any other Council Directive applicable to such waters as regards the presence of mercury. - The concentration of mercury in sediments or in shellfish must not increase significantly with time. - Where several quality objectives are applied to waters in an area, the quality of the waters must be sufficient to meet each of them. - The numerical values of the quality objectives specified in 1.2, 1.3 and 1.4 may, as an exception and where this is necessary for technical reasons, be multiplied by 1.5 until 30 June 1986, provided that the Commission has been notified beforehand. ANNEX III Reference method of measurement - The reference method of analysis for determining the mercury content in waters, the flesh of fish, sediments and shellfish is by flameless atomic absorption spectrophotometry after suitable pre-treatment of the sample which takes account in particular of pre-oxidation of the mercury and of successive reduction of the mercury ions Hg (II). The limits of detection 1 must be such that the mercury concentration can be measured to an accuracy 1 of +/- 30% and a precision 1 of +/- 30% at the following concentrations: - in the case of discharges, one tenth of the maximum permitted concentration of mercury specified in the authorization, - in the case of surface water, one tenth of the mercury concentration specified in the quality objective, - in the case of the flesh of fish and shellfish, one tenth of the mercury concentration specified in the quality objective, - in the case of sediments, one tenth of the mercury concentration in the sample or 0.05 mg/kg dry weight, whichever is the greater. - Flow measurement must be carried out to an accuracy of +/- 20%. 1 The definitions of these terms are as given in Council Directive 79/869/EEC of 9 October 1979 concerning the methods of measurement and frequencies of sampling and analysis of surface water intended for the abstraction of drinking water in the Member States (OJ No L 271, 29.10.1979, p.44). ANNEX IV Monitoring procedure for quality objectives - For each authorization granted in pursuance of this Directive, the competent authority shall specify the restrictions, the monitoring procedure and deadlines for ensuring compliance with the quality objective or objectives concerned. - In accordance with Article 6(3) of Directive 76/464/EEC, the Member State shall report to the Commission for each quality objective chosen and applied, on: - the points of discharge and the means of dispersal, - the area in which the quality objective is applied, - the location of sampling points, - the frequency of sampling, - the methods of sampling and of measurement, - the results obtained. - Samples must be properly representative of the quality of the aquatic environment in the area affected by the discharges, and the frequency of sampling must be sufficient to show any changes in the aquatic environment, taking into account in particular natural variations in the hydrological regime. The salt-water fish analysis must be carried out on a sufficiently representative number of samples and species. - With regard to the quality objective in 1.1 of Annex II, the competent authority shall choose the species of fish to be adopted as indicators for analysis. For salt waters the species chosen from among those inhabiting coastal waters and caught locally may include cod, whiting, plaice, mackerel, haddock and flounder. Statement on Article 3 (3) The Council and the Commission state that the application of the best technical means available makes it possible to limit discharges of mercury from the site of a new industrial plant using the recycled-brine process to less than 0.5 g/tonne of installed chlorine production capacity. |  |  |  |  |  |