| RADA Směrnice rady z 31. března 1982 o sbližování zákonů Členských států vztahujících se k metodám zkoušení biologické odbouratelnosti neiontových povrchově aktivních činidel a doplňující Směrnici 73/404/EHS 82/242/EHS (OJ L 109, 22.4.1982, s.1) Rada Evropských společenství, se zřetelem k Úmluvě ustavující Evropské hospodářské společenství a zvláště na její článek 100, s se zřetelem na návrh Komise 1), se zřetelem na postoj Evropského parlamentu 2), se zřetelem na postoj Hospodářského a sociálního výboru 3), vzhledem k tomu, že zkušební metody platné v členských státech, i když sledující tentýž cíl, se liší v jistých ohledech a jsou tedy rozhodující pro vlastní činnost společného trhu; vzhledem k tomu, že Směrnice Rady 73/404/EHS 4) z 22. listopadu 1973 o sbližování zákonů členských států týkajících se povrchově aktivních látek předpokládá v článku 4 přijetí Směrnice ke stanovení specifických zkušebních metod a také důležitých tolerancí umožňujících splnění požadavků Směrnice; vzhledem k tomu, že Směrnice Rady 73/405/EHS 5) z 22. listopadu 1973 o sbližování zákonů členských států týkající se zkušebních metod odbouratelnosti aniontových povrchově aktivních činidlech specifikovala takové metody a tolerance u aniontových povrchově aktivních činidel; vzhledem k tomu, že k umožnění členským státům určovat hladinu biologické odbouratelnosti neiontových povrchově aktivních činidel je vhodné používat těch zkušebních metod, kterých se již k tomuto účelu v určitých členských státech používá, avšak v případě sporů, by se biologická odbouratelnost zkoušela společnou referenční metodou; 1) OJ C 104, 28.4. 1980, s.112 2) OJ C 197, 4.8.1980, s.66 3) OJ C 310, 30.11.1981, s.7 4) OJ L 347, 17.12.1973, s.51 5) OJ L347,17.12.1973, s.53 vzhledem k tomu, že s ohledem na sbližování zákonů členských států týkajících se povrchově aktivních látek by měly být stanoveny vhodné tolerance k měření biologické odbouratelnosti, jak předpokládá článek 4 ve Směrnici Rady 73/404/EHS, aby se dosáhlo ochrany před nespolehlivostí zkoušek, která by mohla vést k zamítnutí rozhodnutí majících značné hospodářské důsledky; vzhledem k tomu, že k zamítavé rozhodnutí lze vydat pouze tehdy, jestliže výsledky obdržené analytickou metodou citovanou v článku 2 ukáží hladinu biologické odbouratelnosti nižší než 80%; vzhledem k tomu, že v současné době musí být používáno k některým účelům malých množství neiontových povrchově aktivních činidel s nízkou biologickou odbouratelností z důvodu technických problémů a také pro zábranu nežádoucích účinků na zdraví a životní prostředí; vzhledem k tomu, že bude přesto nutné mít příležitost nabýt přehled o používání těchto povrchových činidel s nízkou biologickou odbouratelností z hlediska technického pokroku; vzhledem k tomu, že technický pokrok potřebuje rychlou adaptaci technických požadavků specifikovaných ve Směrnicích o povrchově aktivních látkách; vzhledem k tomu, že ke snazšímu zavádění nutných opatření k dosažení toho by měl být stanoven postup pro těsnou spolupráci mezi členskými státy a Komisí prostřednictvím Výboru pro adaptaci na technický pokrok u Směrnic při odstraňování technických bariér v obchodu s povrchově aktivními látkami. PŘijala tuto směrnici: Článek 1 Tato Směrnice se týká zkušebních metod biologické odbouratelnosti neiontových povrchově aktivních činidel přítomných v povrchově aktivních látkách definovaných v článku 1 Směrnice 73/404/EHS. Článek 2 V souladu s ustanoveními článku 4 Směrnice 73/404/EHS členské státy zakáží na svých územích uvádění na trh a používání povrchově aktivních látek, jestliže hladina biologické odbouratelnosti neiontových povrchově aktivních činidel v nich obsažených je nižší než 80% a určená v souhlasu s jednou z uvedených metod: - metoda OECD uvedená v technická zprávě OECD z 11. června 1976 „Navrhovaná metoda k určení biologické odbouratelnosti povrchově aktivních činidel použitých v syntetických povrchově aktivních látkách“, - metoda používaná v Německu zavedená nařízením „Verordnung über die Abbaubarkeit anionischer und nichtionischer grenzflächenaktiver Stoffe in Wash und Reinigungsmitteln“ z 30. ledna 1977, publikovaném v Bundesgesetablatt, 1977, část I, str. 244, vydaná v předpisech doplňujících předpisy z 18. června 1980, publikované v Bundesgesetzblatt, 1980, část i, strana 706, - metoda používaná ve Francii, schválená dekretem z 28. prosince 1977 publikovaném v Journal officiel de la République Francaise z 18. ledna 1978 a experimentální norma T 73-270, březen 1974, publikované organizací Association francaise de normalisation (AFNOR), - metoda používaná ve Spojeném království zvaná „Porous Pot Test“ (zkouška v porézním hrnci), popsaná ve zprávě Technical Report No. 70/1978 of the Water Research Centre. Článek 3 Na základě postupu stanoveného v článku 5(2) Směrnice 73/404/EHS, názor laboratoře na neiontové povrchově aktivní činidla bude spočívat na referenční metodě (ověřovací zkoušky) popsané v Příloze k této Směrnici. Článek 4 Doplňky požadované k adaptaci Přílohy na technický pokrok budou schvalovány v souladu s postupem stanoveným v článku 7b Směrnice 73/404/EHS. Článek 5 Text byl zahrnut do Směrnice 73/404/EHS. Článek 6 - Členské státy uplatní opatření k dodržení této Směrnice v období 18 měsíců po jejím vyhlášení 6). Budou o tom ihned informovat Komisi. - Členské státy sdělí Komisi texty ustanovení v národním zákoně, který schválí v oblasti pokryté touto Směrnicí. 6) Směrnice byla oznámena Členským státům 8. dubna 1982. Článek 7 Tato Směrnice je určena členským státům. Brusel, 31. března 1982. za Radu předseda P. de Keersmaeker PŘÍLOHA URČOVÁNÍ BIOLOGICKÉ ODBOURATELNOSTI NEIONTOVÝCH POVRCHOVĚ AKTIVNÍCH ČINIDEL Referenční metoda (ověřovací zkouška) KAPITOLA 1 - Definice Neiontová povrchově aktivní činidla ve smyslu této Směrnice jsou ta povrchově aktivní činidla, která po průchodu kationtovým a aniontovým iontoměničem jsou stanovena jako vizmutaktivní látky (BiAS) podle analytického postupu popsaného v kapitole 3. - Potřebné zařízení k měření Měřící metoda používá malého aktivačního kalového zařízení, viz obr. 1 a ve větším detailu na obr. 2. Zařízení sestává ze zásobní nádoby A pro syntetickou odpadní vodu, dávkovacího zařízení B, provzdušňovací nádoby C, usazovací nádoby D, vzduchového čerpadla E k recyklaci aktivizovaného kalu a z nádoby F ke sběru zpracovaného výtoku. Nádoby A a F musejí být za skla nebo z vhodného plastu a objemu aspoň 24 litrů. Zařízení B musí dodávat konstantní tok syntetické odpadní vody do provzdušňovací nádoby; Tato nádoba, během normálního provozu, obsahuje tři litry smíšené kapaliny. Slinutá provzdušňovací kostka G je zavěšena v nádobě C na vrcholu kužele. Množství vzduchu procházejícího provzdušňovačem se monitoruje pomocí průtokoměru H. - Syntetická odpadní voda Ke zkoušce se používá syntetická odpadní voda. Na každý litr vody z vodovodního kohoutku se rozpustí: 160 mg peptonu, 110 mg masového extraktu, 30 mg močoviny CO(NH2)2, 7 mg chloridu sodného NaCl, 4 mg chloridu vápenatého, CaCl2.2H2O, 2 mg síranu hořečnatého, MgSO4, 28 mg hydrofosforečnanu dvojdraselného K2HPO4 a 10 + 1 g BiAS BiAS se extrahuje ze zkoušeného výrobku metodou uvedenou v kapitole 2. Syntetická odpadní voda se připravuje denně čerstvá. - Příprava vzorků - Jednotlivá povrchově aktivní činidla se zkoumají v původním stavu. Obsah BiAS se musí určit, aby se připravila syntetická odpadní voda (1.3). - Vytvořené výrobky se analyzují na obsah BiAS, MBAS a mýdla. Musejí být předmětem alkoholické extrakce a separace substance BiAS (viz kap. 2). Obsah BiAS v extraktu musí být znám, aby se připravila syntetická odpadní voda. - Provoz zařízení Napřed se naplní provzdušňovací nádoba C a usazovací nádoba D syntetickou odpadní vodou. Výška nádoby D se seřídí tak, aby objem v provzdušňovací nádobě byl tři litry. Očkování se provede zavedením 3 ml sekundárního výtoku dobré kvality, čerstvě odebraného z úpravárenského závodu zpracovávajícího převážně odpadní vody z domácností. Výtok musí být přechován za aerobních podmínek v období mezi odběrem vzorku a použitím. Potom se uvedou provzdušňovač G, vzduchové čerpadlo E a dávkovací zařízení B do provozu. Syntetická odpadová voda musí procházet provzdušňovací nádobou C v množství 1 l/h; což dává dobu zdržení tři hodiny. Míra provzdušnění by se měla regulovat tak, aby obsah nádoby C se udržoval v konstatní suspensi a obsah rozpuštěného kyslíku byl aspoň 2 mg/l. Pěnění musí být zabráněno vhodnými prostředky. Činidel proti pěnění, která jsou na překážku aktivaci kalu nebo obsahují BiAS, nesmí být použito. Vzduchové čerpadlo E musí být nastaveno tak, že aktivovaný kal z usazovací nádoby D kontinuálně a pravidelně cirkuluje do provzdušňovací nádoby C. Kal, který se nashromáždil v horní části provzdušňovací nádoby C, na dně usazovací nádoby D nebo v cirkulačním okruhu, se musí vrátit do cirkulace nejméně jednou za den použitím kartáče nebo jiných vhodných prostředků. Když se kal neusazuje, může se jeho usaditelnost zvětšit přidáváním dávek 2 ml pětiprocentního roztoku chloridu železitého, je-li třeba, opakovaně. Výtok z usazovací nádoby D se shromažďuje v nádobě F po dobu 24 hodin; potom se odebere vzorek po důkladném míchání. Nádoba F se pak musí pečlivě vymýt. - Kontrola měřícího zařízení Obsah BiAS (mg/l) syntetické odpadní vody se určuje bezprostředně před použitím. Obsah BiAS (v mg/l) ve výtoku nashromážděném za 24 hodin v nádobě F se určí analyticky toutéž metodou, bezprostředně po nashromáždění; jinak se vzorky musejí uchovávat přednostně zmrazené. Koncentrace musejí být určeny se zaokrouhlením na 0,1 mg/l BiAS. Pro kontrolu účinnosti procesu se měří aspoň dvakrát za týden chemická spotřeba kyslíku (CHSK) nebo rozpuštěný organický uhlík (DOC) ve výtoku, shromážděném v nádobě F, přes filtr se skleněnou vatou a ve filtrované syntetické odpadové vodě. Snížení CHSK a DOC se vyrovná, když se dosáhne zhruba pravidelného denního odbourávání BiAS ke konci spouštěcího období, viz obr. 3. Obsah sušiny v aktivovaném kalu, obsaženém v provzdušňovací nádobě, se určuje v g/l dvakrát týdně. Jestliže je více než 2,5 g/l, přebytek aktivovaného kalu musí být odstraněn. Odbourávací zkouška se provádí při pokojové teplotě; ta by měla být stálá a udržována v rozmezí 292-297 K (19-24°C). - Výpočet biologické odbouratelnosti Procento odbourání BiAS se musí vypočítávat každý den na základě obsahu BiAS v mg/l syntetické odpadní vody a odpovídajícího výtoku kumulovaného v nádobě F. Tak obdržené hodnoty odbouratelnosti se graficky znázorní jako na obr. 3. Odbouratelnost BiAS se vypočte jako aritmetický střed z hodnot obdržených v průběhu 21 dní následujících po spouštěcím období, kdy se odbourávání stalo pravidelným a provoz zařízení je bez poruchy. V každém případě, trvání spouštěcího období by nemělo přesáhnout šest týdnů. Denní hodnoty odbourávání se zaokrouhlují na 0,1%, ale konečný výsledek se zaokrouhluje na nejbližší celé číslo. V některých případech se smí připustit snížení četnosti odběru vzorků, ale nejméně 14 výsledků, shromážděných během 21 dne po spouštěcím období, by se mělo použít při výpočtu průměru. KAPITOLA 2 Předběžná úprava zkoušených výrobků - Předběžné poznámky - Úprava vzorků Úprava neiontových povrchově aktivních činidel a vytvořených povrchově aktivních látek před určováním biologické odbouratelnosti v ověřovací zkoušce je: |  |  | COUNCIL COUNCIL DIRECTIVE of 31 March 1982 on the approximation of the laws of the Member States relating to methods of testing the biodegradability of non-ionic surfactants and amending Directive 73/404/EHS 82/242/EHS (OJ L 109, 22.04.1982, p.1) THE COUNCIL OF THE EUROPEAN COMMUNITIES, Having regard to the treaty establishing the European Economic Community, and in particular Article 100 thereof, Having regard to the proposal from the Commission 1), Having regard to the opinion of the European Parliament 2), Having regard to the opinion of the Economic and Social Committee 3), Whereas the methods of testing in force in the Member States, while pursuing the same objective, differ in certain respects and are thus detrimental to the proper functioning of the common market; Whereas Council Directive 73/404/EHS 4) of 22 November 1973 on the approximation of the laws of the Member States relating to detergents provides in Article 4 for the adoption of Directives specifying methods of testing as well as relevant tolerances to enable compliance with the requirements of that Directive to be established; whereas Council Directive 73/405/EHS 5) of 22 November 1973 on the approximation of the laws of the Member States relating to methods of testing the biodegradability of anionic surfactants specified such methods and tolerances for anionic surfactants; Whereas, to enable Member States to determine the level of biodegradability of non-ionic surfactants, it is advisable to employ methods of testing already in use for this purpose in certain Member States; whereas, however, in the event of disputes, biodegradability should be tested by a common reference method; 1) OJ C 104, 28.4.1980, p.112 2) OJ C 197, 4.8.1980, p.66 3) OJ C 310, 30.11.1981, p.7 4) OJ L 347, 17.12.1973,p.51 5) OJ L 347, 17.12.1973, p.53 Whereas, in regard to the approximation of the laws of the Member States relating to detergents, suitable tolerances for measuring biodegradability should be laid down, as provided for in Article 4 of Council Directive 73/404/EHS, in order to safeguard against the unreliability of methods of testing which could lead to rejection decisions having considerable economic consequences; whereas a rejection decision must be taken only if the results obtained by an analytical method mentioned in Article 2 show a level of biodegradability lower than 80%; Whereas for the time being small quantities of certain non-ionic surfactants of low biodegradability must be used for some purposes because of technical problems and in order to prevent other undesirable effects on health and the environment; whereas it will nevertheless be necessary to have the opportunity of reviewing the use of these surfactants of low biodegradability in the light of technical progress; Whereas technical progress necessitates a rapid adaptation of the technical requirements specified by the Directives on detergents; whereas, to facilitate implementation of the necessary measures designed to achieve this, a procedure should be established providing for close collaboration between the Member States and the Commission by means of a Committee for the adaptation to technical progress of the Directives on the removal of technical barriers to trade in detergents, HAS ADOPTED THIS DIRECTIVE: Article 1 This Directive concerns the methods of testing the biodegradability of non-ionic surfactants present in the detergents defined in Article 1 of Directive 73/404/EHS. Article 2 In accordance with the provisions of Article 4 of Directive 73/404/EHS, the Member States shall prohibit the placing on the market and use on their territory of a detergent if the level of biodegradability of the non-ionic surfactants contained in such detergent is less than 80% determined in accordance with one of the following methods: - the OECD method, published in the OECD technical report of 11 June 1976 on the "proposed method for the determination of the biodegradability of surfactants used in synthetic detergents", - the method in use in Germany, established by the "Verordnung ueber die Abbaubarkeit anionischer und nichtionischer grenzflaechenaktiver Stoffe in Wasch - und Reinigungsmitteln" of 30 January 1977, published in the Bundesgesetzblatt, 1977, Part I, page 244, as set out in the Regulation amending that Regulation of 18 June 1980, published in the Bundesgesetzblatt, 1980, Part I, page 706, - the method in use in France, approved by decree of 28 December 1977 published in the Journal officiel de la Republique francaise of 18 January 1978, and experimental standard T 73-270 March 1974, published by the Association francaise de normalisation (AFNOR), - the method in use in the United Kingdom called the "porous pot test", as described in Technical Report No 70/1978 of the Water Research Centre. Article 3 Under the procedure laid down in Article 5(2) of Directive 73/404/EHS, the laboratory opinion on non-ionic surfactants shall be based on the reference method (confirmatory test procedure) described in the Annex to this Directive. Article 4 The amendments required for adapting the annex to technical progress shall be adopted in accordance with the procedure laid down in Article 7b of Directive 73/404/EHS. Article 5 This text has been inserted in Directive 73/404/EHS. Article 6 - The Member States shall bring into force the provisions necessary to comply with this Directive within a period of 18 months following its notification6). They shall forthwith inform the Commission thereof. - Member states shall communicate to the Commission the text of the provisions of national law which they adopt in the field covered by this Directive. 6) This Directive was notified to the Member States on 8 April 1982. Article 7 This Directive is addressed to the Member States. Done at Brussels, 31 March 1982. For the Council The President P. de Keersmaeker ANNEX DETERMINATION OF THE BIODEGRADABILITY OF NON-IONIC SURFACTANTS Reference method (confirmatory test) CHAPTER 1 - Definition Non-ionic surface active agents in the sense of this Directive are those surface active agents which, after passage through cationic and anionic ion exchangers, are determined as bismuth-active substance (BiAS) according to the analytical procedure described in Chapter 3. - Equipment needed for measurement The method of measurement employs the small activated sludge plant shown in Figure 1, and in greater detail in Figure 2. The equipment consists of a storage vessel A for synthetic sewage, dosing pump B, aeration vessel C, settling vessel D, air-lift pump E to recycle the activated sludge, and vessel F for collecting the treated effluent. Vessels A and F must be of glass or suitable plastic and hold at least 24 litres. Pump B must provide a constant flow of synthetic sewage to the aeration vessel; this vessel, during normal operation, contains three litres of mixed liquor. A sintered aeration cube G is suspended in the vessel C at the apex of the cone. The quantity of air blown through the aerator should be monitored by means of a flowmeter H. - Synthetic sewage A synthetic sewage is employed for the test. Dissolve in each litre of tap water: 160 mg peptone, 110 mg meat extract, 30 mg urea CO(NH2)2, 7 mg sodium chloride NaCl, 4 mg calcium chloride, CaCl2.2H2O, 2 mg magnesium sulphate, MgSO4.7H2O, 28 mg of dipotassium hydrogen phosphate K2HPO4, and 10 + 1 mg BiAS. The BiAS is extracted from the product to be tested by the method Given in Chapter 2. The synthetic sewage is freshly prepared daily. - Preparation of samples - Uncompounded surfactants may be examined in the original state. The bias content must be determined in order to prepare the synthetic sewage (1.3). - Formulated products are analyzed for BiAS, MBAS and soap content. They must be subjected to an alcoholic extraction and to a separation of the BiAS(see Chapter 2). The BiAS content of the extract must be known in order to prepare the synthetic sewage. - Operation of equipment Initially, fill aeration vessel C and settling vessel D with synthetic sewage. The height of the vessel D should be so fixed that the volume contained in the aeration vessel C is three litres. Inoculation is made by introducing 3 ml of a secondary effluent of good quality, freshly collected from a treatment plant dealing with a predominantly domestic sewage. The effluent must be kept under aerobic conditions in the period between sampling and application. Then set the aerator G, air-lift E and dosing device B in operation. The synthetic sewage must pass through the aeration vessel C at a rate of one litre per hour; this gives a mean retention time of three hours. The rate of aeration should be so regulated that the contents of vessel C are kept constantly in suspension and the dissolved oxygen content is at least 2 mg/l. Foaming must be prevented by appropriate means. Anti-foaming agents which inhibit the activated sludge or contain BiAS must not be used. The air-lift pump E must be set so that the activated sludge from the settling vessel is continually and regularly recycled to aeration vessel C. Sludge which has accumulated around the top of the aeration vessel C, in the base of the settling vessel D or in the circulation circuit must be returned to the circulation at least once each day by brushing or some other appropriate means. When the sludge fails to settle, its settlebility may be increased by the addition of 2 ml portions of a 5% solution of ferric chloride, repeated as necessary. The effluent from the settling vessel D is accumulated in vessel f for 24 hours, following which a sample is taken after thorough mixing. Vessel F must then be carefully cleaned. - Checking measuring equipment The BiAS content (in mg/l) of the synthetic sewage is determined immediately before use. The BiAS content (in mg/l) of the effluent collected over 24 hours in vessel F should be determined analytically by the same method, immediately after collection: otherwise the samples must be preserved, preferably by freezing. The concentrations must be determined to the nearest 0.1 mg/l BiAS. As a check on the efficiency of the process, the chemical oxygen demand (COD) or the dissolved organic carbon (DOC) of the glass fibre filtered effluent accumulated in vessel F and of the filtered synthetic sewage in vessel A is measured at least twice per week. The reduction in COD or DOC should level off when a roughly regular daily BiAS degradation is obtained at the end of the running-in period shown in Figure 3. The content of dry matter in the activated sludge contained in the aeration vessel should be determined twice a week in g/l. If it is more than 2.5 g/l, the excess activated sludge must be discarded. The degradation test is performed at room temperature; this should be steady and kept between 292 and 297 K (19-24°C). - Calculation of biodegradability The percentage degradation of BiAS must be calculated every day on the basis of the BiAS content in mg/l of the synthetic sewage and of the corresponding effluent accumulated in vessel F. The degradability figures thus obtained should be presented graphically as in Figure 3. The degradability of the BiAS should be calculated as the arithmetic mean of the figures obtained over the 21 days which follow the running-in period, during which degradation has been regular and the operation of the plant trouble-free. In any event the duration of the running-in period should not exceed six weeks. The daily degradation values are calculated to the nearest 0.1% but the final result is given to the nearest whole number. In some cases it may be permissible to reduce the frequency of sampling but at least 14 results collected over the 21 days which follow the running-in period should be used in calculating the average. CHAPTER 2 Preliminary treatment of products to be tested - Preliminary notes - Treatment of samples The treatment of non-ionic surface active agents and formulated detergents prior to the determination of biodegradability in the Confirmatory test is: |
| --- | --- | --- | --- |
| Výrobky - Products |  | Úprava - Treatment |  |
| Neiontové povrchově aktivní činidla Non-ionic surfactants |  | Žádná None |  |
| Vytvořené povrchově aktivní látky Formulated detergents |  | Extrakce alkoholem s následnou separací neiontových povrchově aktivních činidel iontovou výměnou Alcoholic extraction followed by separation of the non-ionic surfactants by ion exchange |  |
| Účelem extrakce alkoholem je vyloučit nerozpustné a anorganické příměsi v komerčním výrobku, které za některých okolností mohou zvrátit zkoušku biologické odbouratelnosti. - Iontová výměna Pro správnost zkoušky biologické odbouratelnosti se požaduje izolace a separace neiontových, povrchově aktivních činidel od mýdla, a iontových a kationtových povrchově aktivních činidel. To se dosahuje technikou iontové výměny při použití makroporézní pryskyřice a vhodných vypíracích prostředků pro frakční vypírání. Tedy mýdlo, aniontové a neiontové povrchově aktivní činidla mohou být separovány při jednom postupu. - Analytická kontrola Po homogenizaci, koncentrace aniontových a neiontových povrchově aktivních činidel v povrchově aktivních látkách se určí analytickým postupem MBAS a BiAS. Obsah mýdla se určí vhodnou analytickou metodou. Analýza výrobku je nutná k výpočtu množství žádaných pro přípravu frakcí ke zkouškám biologické odbouratelnosti. Kvantitativní extrakce není nutná, ale nejméně 80% neiontových povrchově aktivních činidel by mělo být extrahováno. Obvykle se dosáhne 90% nebo více. - Princip Z homogenního vzorku (prachy, sušené mazy a sušené kapaliny) se získává etanolový extrakt, který obsahuje povrchově aktivní činidla, mýdlo a jiné, v alkoholu rozpustné, složky vzorku povrchově aktivní látky. Etanolový extrakt se odpaří do sucha, rozpustí se ve směsi izopropanol-voda a získaný roztok prochází přes kombinaci silně kyselý katex/makroporézní anex, zahřátou na 323 K (50°C). Tato vysoká teplota je nutná, aby se zabránilo vysrážení různých mastných kyselin, které mohou být přítomné v kyselém médiu. Neiontová povrchově aktivní činidla se získají odpařením výtoku. Kationtová povrchově aktivní činidla, která by mohla zvrátit zkoušku odbouratelnosti a analytických postupů jsou odstraněna katexem umístěným před anexem. - Chemikálie a zařízení - Deionizovaná voda - Etanol, 95%obj. C2H5OH, přípustný denaturant: metyletylketon nebo metanol - Směs izopropanol/voda (50/50 objemově): 50 dílů izopropanolu (CH3CHOHCH3) na 50 dílů vody (2.3.1) - Roztok hydrouhličitan amonný (60/40 hm.): 0,3 molu NH4HCO3 v 1 000 ml směsi izopropanol/voda složené z 60 objemových dílů izopropanolu a 40 objemových dílů vody (2.3.1) - Katex, silně kyselý, odolný alkoholu (50-100 mesh) - Anex, makroporézní, Merck Lewatit MP 7080 (70-150 mesh) nebo ekvivalentní - Kyselina chlorovodíková, 10% hm. HCl - Baňka s kulatým dnem 2 000 ml se skleněnou zátkou a zpětným chladičem - Sací filtr o průměru 90 mm (vyhřívatelný) na filtrační papíry - Filtrační baňka 2 000 ml - Výměnné kolony s vyhřívacím pláštěm a kohoutem, vnitřní trubka průměr 60 mm a výška 450 mm (obr. 4) - Vodní lázeň - Vakuová sušicí pec - Termostat - Rotační odparka - Příprava extraktu a separace neiontových povrchově aktivních činidel - Příprava extraktu Množství povrchově aktivního činidla potřebné ke zkoušce odbourávání je kolem 25 g BiAS. Při přípravě extraktů pro zkoušky odbourávání je množství použitého výrobku omezeno na max. 2 000 g. Proto může být třeba vykonat operaci dvakrát nebo vícekrát, aby se dostalo postačující množství na zkoušky odbourávání. Zkušenost ukázala, že je výhodně použít raději více malých extrakcí než jednu velkou extrakci. - Izolace složek rozpustných a alkoholu 250 g syntetické povrchově aktivní látky, která se bude analyzovat, se přidá k 1 250 ml etanolu a směs se ohřeje na bod varu. Vaří se jednu hodinu pod zpětným chladičem za míchání. Horký alkoholický roztok se rychle zfiltruje hrubě porézním sacím filtrem ohřátým na 323 K (50°C). Baňka a sací filtr se promyje asi 200 ml horkého etanolu. Filtrát a filtrové promývky se shromáždí ve filtrační baňce. V případě, že se mají analyzovat mazy nebo kapalné výrobky, je třeba se ujistit, že ve vzorku není obsaženo více než 25 g neiontových povrchově aktivních činidel a 35 g mýdla. tento Zvážený vzorek se odpaří k suchu. Odparek se rozpustí v 500 ml etanolu a postupuje se jak uvedeno výše. V případě prachů malé sypné hustoty (< 300 g/l) se doporučuje zvýšit podíl etanolu v poměru 10:1. K odpařování etanolového filtrátu k suchu se přednostně použije rotační odparka. Je-li třeba větší množství extraktu, postup se opakuje. Odparky se rozpustí v 5 000 ml směsi izopropanol/voda. - Příprava ionexových výměnných kolon Katexová kolona 600 ml katexové pryskyřice (2.3.5) se v kádince přelije 2 000 ml kyseliny chlorovodíkové (2.3.7) a nechá se stát nejméně dvě hodiny s příležitostným promícháním. Kyselina se slije a pryskyřice se za pomoci deionizované vody přenese do kolony (2.3.11). Kolona musí mít píst ze skelné vaty. Kolona se promývá deionizovanou vodou rychlostí 10 - 30 ml/min, až je výtok prostý chloridů. Voda se odstraní promytím 2 000 ml směsi izopropanol/voda (2.3.3) rychlostí 10-30 ml/min. Výměnná kolona je tak připravena k provozu. Anexová kolona 600 ml anexové pryskyřice (2.3.6) se v kádince přelije 2 000 ml deionizované vody a nechá se bobtnat nejméně dvě hodiny. Pryskyřice se přenese do kolony pomocí dionizované vody. Kolona musí mít píst ze skelné vaty. Kolona se promývá roztokem 0,3M hydrouhličitanu amonného (2.3.4), až je roztok prostý chloridů. To vyžaduje kolem 5 000 ml roztoku. Opakovaně se promyje 2 000 ml deionizované vody. Voda se odstraní 2 000 ml směsi izopropanol/voda (2.3.2) rychlostí 10-30 ml/min. Výměnná kolona je tak ve formě OH a připravena k provozu. - Postup výměny iontů Výměnné kolony se zapojí tak, aby katexová kolona byla umístěna před anexovou kolonou. Kolony se ohřejí na 323 K (50°C) s použitím termostatického řízení teploty. 5 000 ml roztoku získaného dle 2.4.2 se ohřeje na 333 K (60°C) a nechá se procházet sestavou kolon rychlostí 20 ml/min. Kolony se promyjí 1 000 ml horké směsi izopropanol/voda (2.3.3). K získání neiontových povrchově aktivních činidel se shromáždí filtrát a filtrové promývky a odpaří se k suchu (přednostně na rotační odparce). Odparek obsahuje BiAS. Přidá se deionizovaná voda do definovaného objemu a určí se alikvotní obsah BiAS podle 3.3. Roztoku se použije jako standardního roztoku neiontového povrchově aktivního činidla. Roztok se musí uchovávat při teplotě pod 278 K (5°C). - Regenerace ionexových pryskyřic Katex se po použití odstraní. Anexová pryskyřice se regeneruje v koloně průchodem asi 5 000-6 000 ml roztoku hydrouhličitanu amonného (2.3.4) rychlostí přibližně 10 ml/min, až je výtok prostý neiontového povrchově aktivního činidla (zkouška metylénovou modří). Pak se promyje 2 000 ml směsi izopropanol/voda a anex je tím opět připraven k provozu. KAPITOLA 3 Stanovení neiontových povrchově aktivních činidel ve zkoumaných tekutinách - Princip Povrchově aktivní činidla se koncentrují a izolují stripováním (vypuzováním) plynem. Použitý vzorek by měl obsahovat 250 až 800 g neiontových povrchově aktivních činidel. Vystripovaná povrchově aktivní činidla se rozpustí v octanu etylnatém. Po oddělení fází a odpaření roztoku, se neiontová povrchově aktivní činidla vysráží vodním roztokem modifikovaného Dragendorffova činidla (KBiJ4+BaCl2+ledová kyselina octová). Sraženina se zfiltruje, vymyje ledovou kyselinou octovou a rozpustí v roztoku vinanu amonného. Bismut se titruje potenciometricky v roztoku pyrolidinditiokarbámátu při pH 4-5 s použitím lesklé platinové indikační elektrody a kalomelové nebo stříbrochloridové referenční elektrody. Metoda je použitelná u neiontových povrchově aktivních činidel obsahujících 6-30 alkenoxidových skupin. Výsledek titrace se násobí empirickým faktorem 54 pro konverzi na referenční substanci nonylfenol kondenzovaný s 10 moly etylenoxidu (NP 10). - Přípravky a zařízení Reagencie se připravují v deionizované vodě. - Čistý octan etylnatý, čerstvě destilovaný. - Hydrouhličitan sodný (NaHCO3) p.a. - Zředěná kyselina chlorovodíková (20 ml koncentrované kyseliny (HCl) rozpuštěné v 1 000 ml vody). - Metanol p.a., čerstvě destilovaný, přechovávaný ve skleněné láhvi. - Bromkrezolový purpur, 0,1 g ve 100 ml metanolu. - Srážecí činidlo: směs dvou objemů roztoku A a jednoho objemu roztoku B. Směs se přechovává v hnědé láhvi a může se použít po dobu jednoho týdne od přípravy. - Roztok A 1,7 g zásaditého dusičnanu vizmutitého p.a. (BiONO3.H2O) se rozpustí ve 20 ml ledové kyseliny octové a doplní se vodou na 100 ml. Pak se rozpustí 65 g jodidu draselného p.a. v 200 ml vody. Oba tyto roztoky se smíchají v 1 000 ml odměrné baňce, přidá se 200 ml ledové kyseliny octové (3.2.7) a doplní se vodou na 1 000 ml. - Roztok B 290 g chloridu barnatého (BaCl2.2H2O) p.a.se rozpustí v 1 000 ml vody. - Ledová kyselina octová 99-100% (nižší koncentrace jsou nevhodné). - Roztok vinanu amonného: 12,4 g kyseliny vinné p.a. a 12,4 ml roztoku amoniaku p.a. (d = 0,910 g/ml) (nebo ekvivalentní množství vinanu amonného p.a.) se doplní vodou na 1000 ml. - Zředěný roztok amoniaku: 40 ml amoniaku p.a. (d = 0,910 g/ml) se doplní vodou na 1 000 ml. - Standardní octanový pufr: 40 g hydroxidu sodného p.a.se rozpustí v 500 ml vody v kádince a nechá se zchladit. Přidá se 120 ml ledové kyseliny octové (3.2.7), důkladně se promíchá, zchladí. Po přenesení do 1 000 ml odměrné baňky se doplní vodou k rysce. - Roztok pyrolidinditiokarbámátu (známý jako „karbamátový roztok“): 40 g pyrolidinditiokarbamátu sodného (C5H8NNaS2.2H2O) se rozpustí ca 500 ml vody, přidá se 10 m. n-amylalkoholu p.a. a 0,5 g hydrouhličitanu sodného (NaHCO3) p.a. a doplní se vodou na 1 000 ml. - Roztok síranu měďnatého (pro standardizaci 3.2.11). Zásobní roztok: 1,249 g síranu měďnatého (CuSO4.5H2O) p.a. se smíchá s 50 ml 0,5M kyseliny sírové a doplní se vodou na 1 000 ml. Standardní roztok: 50 ml zásobního roztoku se smíchá s 10 ml 0,5M kyseliny sírové a doplní se vodou na 1 000 ml. - Chlorid sodný p.a.. - Plynová stripovací aparatura (obr. 5). Průměr slinutého kotouče musí být týž jako vnitřní průměr válce - Dělící nálevka 250 ml. - Magnetické míchadlo s magnetem 25-30 mm. - Goochův kelímek, průměr perforované základny 25 mm, typ G 4. - Kruhové filtry z filtračního papíru ze skleněných vláken, průměr 27 mm, průměr vláken 0,5-1,5 m. - Dvě filtrační baňky s adaptory a pryžovými manžetami, po 500 a 250 ml. - Zapisovací potenciometr rozsahu 250 mV, v kompletu s lesklou platinovou indikační elektrodou a kalomelovou nebo Ag/AgCl referenční elektrodou, a s automatickou byretou o obsahu 20-25 ml nebo alternativní ruční zařízení. - Metoda - Koncentrace a separace povrchově aktivních látek. Vodný roztok se zfiltruje kvalitním papírovým filtrem. Prvních 100 ml filtrátu se odstraní. Do plynového stripovací aparatury, předtím vypláchnuté octanem etylnatým, se vloží odměřené množství vzorku s obsahem 250-800 g neiontového povrchově aktivního činidla. K zlepšení separace přidej 100 g chloridu sodného a 5 g hydrouhličitanu sodného. Jestliže obsah vzorku je větší než 500 ml, přidají se tyto soli v tuhém stavu do stripovací aparatury a rozpustí se proháněním dusíku nebo vzduchu. Je-li použito menšího vzorku, soli se rozpustí ve 400 ml vody a pak se přelijí do stripovací aparatury. Doplňuje se voda, až hladina dostoupí k hornímu uzavíracímu kohoutu. Na hladinu vody se opatrně přidá 100 ml octanu etylnatého. Promývací baňka, připojená k vedení plynu (dusík nebo vzduch), se do dvou třetin naplní octanem etylnatým. Průtok plynu aparaturou se nastaví na 30-60 l/h; doporučuje se montáž rotametru. Rychlost provzdušňování se zvyšuje postupně. Množství plynu se nastaví tak, aby fáze byly zřetelně separovány a minimalizováno míšení fází a roztoku octanu etylnatého ve vodě. Proud plynu se zastaví po pěti minutách. Jestliže dojde ke snížení o více než 20% v objemu organické fáze ve prospěch vodného roztoku, odstraňování se musí opakovat se zvláštní pozornosti k rychlosti proudu plynu. Organickou fáze se vypustí do dělící nálevky. Jakákoliv voda z vodní fáze - mělo by to být pouze několik ml - se vrátí do stripovací aparatury. Fáze octanu etylnatého se zfiltruje přes suchý, kvalitní papírový filtr do kádinky 250 ml. Dalších 100 ml octanu etylnatého se vloží do stripovací aparatury a opět se prohání dusík nebo vzduch po dobu 5 minut. Vypustí se organická fáze do dělící nálevky použité při první separaci, odpustí se vodná fáze a organická fáze se zfiltruje přes tentýž filtr jako u první části octanu etylnatého. Dělicí nálevka a filtr se vypláchne množstvím asi 20 ml octanu etylnatého. Extrakt octanu etylnatého se odpaří k suchu ve vodní lázní (digestoř). Aby se odpařování urychlilo, přivádí se mírný proud vzduchu na povrch roztoku. - Srážení a filtrace Odparek z 3.3.1 se rozpustí v 5 ml metanolu, přidá se 40 ml vody a 0,5 ml zředěné kyseliny chlorovodíkové (3.2.3) a směs se promíchá magnetickým míchadlem. Do tohoto roztoku se přidá 30 ml srážecího činidla (3.2.6) z odměrného válce. Sraženina se vytváří po opakovaném míchání. Po 10 minutách míchání se směs ponechá alespoň 5 minut.v klidu. Směs se zfiltruje přes Goochův kelímek, jehož základna je pokryta filtrem z filtračního papíru ze skleněných vláken. Filtr se předem promyje prosátím asi 2 ml ledové kyseliny octové. Pak se důkladně vymyjí kádinka, magnet a kelímek ledovou kyselinou octovou, jíž je třeba asi 40-50 ml. Není nutné kvantitativně přenášet sraženinu lnoucí na stěně kádinky na filtr, poněvadž roztok se sraženinou k titraci se vrací do precipitační kádinky a zbývající sraženina se pak rozpustí. - Rozpuštění sraženiny Sraženina ve filtračním kelímku se rozpustí přidáním horkého roztoku vinanu amonného (asi 80°C, 353 K) (3.2.8) ve třech dílech, každý po 10 ml. Ponechej každý díl v kelímku po několik minut v klidu než se prosákne přes filtr do baňky. Obsah filtrační baňky se přelije do kádinky použité k srážení a stěny kádinky se opláchnou dalším množstvím 20 ml roztoku vinanu k rozpuštění zbytků sraženiny. Kelímek, adaptér a filtrační baňka se pečlivě vymyje 150-200 ml vody, která se a vrátí do kádinky použité k srážení. - Titrace Roztok se míchá magnetickým míchadlem (3.2.16), přidá se roztok rozpuštěného amoniaku (3.2.9) až se barva změní na fialovou (roztok je slabě kyselý po zbytku ledové kyseliny octové použité k vypláchnutí). Potom se přidá 10 ml standardního acetátového pufru, do roztoku se ponoří elektrody a titruje se potenciometricky standardním „karbamátovým roztokem“ (3.2.11), přičemž konec byrety je ponořen do roztoku. Rychlost titrace by neměla překročit 2 ml/min. Konečný stav je průsečík tečen ke dvěma větvím křivky potenciálu. Příležitostně se pozoruje, že křivka potenciálu se stává plošší; tomu s dá zabránit pečlivými očištěním platinové elektrody (vyleštěním smirkovým papírem). - Slepý pokus Souběžně se provede srovnávací (slepé) stanovení s 5 ml metanolu a 40 ml vody s dodržením celého postupu podle 3.3.2. Spotřeba na slepou titraci by měla být menší než 1 ml, jinak při podezření na čistotu reagencií (3.2.3, 3.2.7, 3.2.8, 3.2.9, 3.2.10), zvláště na obsah těžkých kovů, je tyto třeba vyměnit. Slepý pokus se musí vzít do úvahy při výpočtu výsledků. - Kontrola faktoru „karbamátového roztoku“ Faktor karbamátového roztoku se určuje ke dni použití. Za tím účelem se titruje 10 ml roztoku síranu měďnatého (3.2.12) karbamátovým roztokem po přidání 100 ml vody a 10 ml standardního octanového pufru (3.2.10). Jestliže spotřeba je „a“ ml, pak faktor f je: f = 10/a a všechny výsledky titrace musejí být násobeny tímto faktorem. - Výpočet výsledků Každé neiontové povrchově aktivní činidlo má svůj vlastní faktor, který závisí na jeho složení, zvláště na délce alkenoxidového řetězce. Koncentrace neionogenního povrchově aktivního činidla je vyjádřena ve vztahu ke standartní substanci - nonylfenol s 10 jednotkami etylenoxidu (NP 10) - u níž je konverzní faktor 0,054. Při použití tohoto faktoru je množství povrchově aktivního činidla ve vzorku, vyjádřené v mg ekvivalentu NP 10, dáno vzorcem: (b-c) . f . 0,054 = mg neionogenního povrchově aktivního činidla vztažené k NP 10, kde: b = spotřeba „karbamátového roztoku“ při titraci vzorku (ml), c = spotřeba „karbamátového roztoku“ při slepém pokusu (ml). f = faktor „karbamátového roztoku“. - Vyjádření výsledků Výsledek se vyjádří v mg/l NP 10 se zaokrouhlením na 0,1. OBRÁZKY Obrázky č. 1 až 5 - viz Official Journal No 109, 22.4.1982, str.1 |  |  | The purpose of the alcoholic extraction is to eliminate the insoluble and inorganic ingredients of the commercial product, which in some circumstances might upset the biodegradability test. - Ion-exchange procedure Isolation and separation of non-ionic surface active agents from soap, anionic and cationic surfactants is required for correct biodegradability tests. This is achieved by an ion-exchange technique using a macro-porous exchange resin and suitable elutants for fractional elution. Thus soap, anionic and non-ionic surfactants may be isolated in one procedure. - Analytical control After homogenizing, the concentration of anionic and non-ionic surfactants in the detergent is determined according to the MBAS and bias analytical procedure. The soap content is determined by a suitable analytical method. This analysis of the product is necessary to calculate the quantities required to prepare fractions for the biodegradability tests. Quantitative extraction is not necessary; however, at least 80% of the non-ionic surfactants should be extracted. Usually, 90% or more is obtained. - Principle From an homogeneous sample (powders, dried pastes and dried liquids)an ethanol extract is obtained which contains the surfactants, soap and other alcohol-soluble constituents of the detergent sample. The ethanol extract is evaporated to dryness, dissolved in an isopropanol/water mixture and the solution obtained is passed through a strongly acidic cation exchange/macroporous anion exchange combination heated to 323 K (50°C). This high temperature is necessary to prevent the precipitation of any fatty acids, which may be present in acidic media. The non-ionic surfactants are obtained from the effluent by evaporation. Cationic surfactants, which might upset the degradation test and the analytical procedure are eliminated by the cation exchanger placed above the anion exchanger. - Chemicals and equipment - Deionized water - Ethanol, 95% (v/v) C2H5OH (permissible denaturant: methyl-ethyl ketone or methanol) - Isopropanol/water mixture (50/50 v/v): 50 parts by volume isopropanol (CH3CHOH.CH3) and 50 parts by volume water (2.3.1) - Ammonium bicarbonate solution (60/40 v/v): 0.3 mol NH4HCO3 in 1 000 ml of an isopropanol/water mixture consisting of 60 parts by volume isopropanol and 40 parts by volume water (2.3.1) - Cation exchanger (CAT), strongly acidic, resistant to alcohol (50-100 mesh) - Anion exchanger (AAT), macro-porous, Merck Lewatit MP 7080 (70-150 mesh) or equivalent - Hydrochloric acid, 10% HCl (w/w) - 2 000 ml round-bottomed flask with ground glass stopper and reflux condenser - 90 mm diameter suction filter (heatable) for filter papers - 2 000 ml filter flask - Exchange columns with heating jacket and tap: inner tube 60 mm in diameter and 450 mm in height (figure 4) - Water-bath - Vacuum drying oven - Thermostat - Rotary evaporator - Preparation of extract and separation of non-ionic active agents - Preparation of extract The quantity of surface active agents necessary for the degradation test is about 25 g BiAS. In preparing extracts for the degradation tests, the quantity of product to be used should be limited to a maximum of 2 000 g. Therefore it may be necessary to carry out the operation two or more times in order to obtain sufficient quantity for the degradation tests. Experience has shown that there are advantages in using a number of small extractions rather than one large extraction. - Isolation of alcohol-soluble constituents Add 250 g of the synthetic detergent to be analyzed to 1 250 ml ethanol and heat the mixture to boiling point and reflux for one hour with stirring. Pass the hot alcoholic solution through a coarse-pored suction filter heated to 323 K (50°C) and filter rapidly. Wash the flask and suction filter with approximately 200 ml hot ethanol. Collect the filtrate and filter washings in a filter flask. In the case of pastes or liquid products to be analyzed, make sure that not more than 25 g anionic surfactants and 35 g soap are contained in the sample. Evaporate this weighed sample to dryness. Dissolve the residue in 500 ml ethanol and proceed as described above. In case of powders of low apparent density (< 300 g/l) it is recommended to increase the ethanol ratio in the relation 20:1. Evaporate the ethanolic filtrate to complete dryness, preferably by means of rotary evaporator. Repeat the operation if a greater quantity of extract is required. Dissolve the residue in 5 000 ml isopropanol/water mixture. - Preparation of ion-exchange columns Cation-exchange column Place 600 ml cation-exchange resin (2.3.5) in a 3 000 ml beaker and cover by adding 2 000 ml hydrochloric acid (2.3.7). Allow to stand for at least two hours, with occasional stirring. Decant the acid and transfer the resin into the column (2.3.11) by means of deionized water. The column should contain a glass wool plug. Wash the column with deionized water at a rate of 10-30 ml/min until the eluate is free of chloride. Displace the water with 2 000 ml isopropanol/water mixture (2.3.3) at a rate of 10-30 ml/min. The exchange column is now ready for operation. Anion-exchange column Place 600 ml anion-exchange resin (2.3.6) in a beaker and cover by adding 2 000 ml deionized water. Allow the resin to swell for at least two hours. Transfer the resin into the column by means of deionized water. The column should contain a glass wool plug. Wash the column with 0.3M ammonium bicarbonate solution (2.3.4) until free of chloride. This requires about 5 000 ml solution. Wash again with 2 000 ml deionized water. Displace the water with 2 000 ml isopropanol/water mixture (2.3.3) at a rate of 10-30 ml/min. The exchange column is now in the oh form and ready for operation. - Ion-exchange procedure Connect the exchange columns so that the cation-exchange column is placed on top of the anion-exchange column. Heat the exchange columns to 323 K (50°C) using thermostatic control. Heat 5 000 ml of the solution obtained in item 2.4.2 to 333 k (60°C) and pass the solution through the exchanger combination at a rate of 20 ml/min. Wash the columns with 1 000 ml hot isopropanol/water mixture (2.3.3). To obtain the non-ionic surface active agents, collect the filtrate and filter washings and evaporate to dryness, preferably by means of a rotary evaporator. The residue contains the bias. Add deionized water until a defined volume is obtained and determine the BiAS content as in item 3.3 in an aliquot. The solution is used as a standard solution of non-ionic surfactants for the degradation test. The solution should be kept at a temperature below 278 K (5°C). - Regeneration of ion-exchange resins The cation exchanger is rejected after use. The anion-exchange resin is regenerated by passing about 5 000-6 000 ml of ammonium bicarbonate solution (2.3.4) down the column at a flow rate of approximately 10 ml/min until the eluate is free from anionic surfactants (methylene blue test). Then pass 2 000 ml isopropanol/water mixture (2.3.3) down the anion exchanger to wash. The anion exchanger is again ready for operation. CHAPTER 3 Determination of non-ionic surface active agent in biodegradation test liquors - Principle Surface active agents are concentrated and isolated by gas stripping. In the sample used, the quantity of non-ionic surfactant should be in the range 250-800 g. The stripped surfactant is dissolved in ethyl acetate. After phaser separation and evaporation of the solvent, the non-ionic surfactant is precipitated in aqueous solution with modified Dragendorff reagent (BiJ4 + BaCl2 + glacial acetic acid). The precipitate is filtered, washed with glacial acetic acid and dissolved in ammonium tartrate solution. The bismuth in the solution is titrated potentiometrically with pyrrolidinedithiocarbamate solution at pH 4-5 using a bright platinum indicator electrode and a calomel or silver/silver chloride reference electrode. The method is applicable to non-ionic surfactants containing 6-30 alkylene oxide groups. The titration result is multiplied by the empirical factor of 54 for conversion to the reference substance nonylphenol condensed with 10 mols ethylene oxide (NP 10). - Reagents and equipment Reagents are to be made up in deionized water. - Pure ethyl acetate, freshly distilled. - Sodium bicarbonate (NaHCO3) AR - Dilute hydrochloric acid (20 ml concentrated acid (HCl) diluted to 1 000 ml with water) - Methanol AR, freshly distilled, stored in a glass bottle. - Bromocresol purple, 0.1 g in 100 ml methanol. - Precipitating agent: the precipitating agent is a mixture of two volumes of solution A and one volume of solution B. The mixture is stored in a brown bottle and can be used for up to one week after mixing. - Solution A Dissolve 1,7 g bismuth nitrate AR (BiONO3.H2O) in 20 ml glacial acetic acid, and make up to 100 ml with water. Then dissolve 65 g potassium iodide AR in 200 ml water. Mix these two solutions in a 1 000 ml measuring flask, add 200 ml glacial acetic acid (3.2.7) and make up to 1 000 ml with water. - Solution B Dissolve 290 g barium chloride (BaCl2.2H2O) AR in 1 000 ml of water. - Glacial acetic acid 99-100% (lower concentrations are unsuitable). - Ammonium tartrate solution : mix 12.4 g tartaric acid AR and 12.4 ml of ammonia solution AR (d = 0.910 g/ml) and make up to 1 000 ml with water (or use the equivalent amount of ammonium tartrate AR). - Dilute ammonia solution : 40 ml ammonia solution AR (d = 0.910 g/ml) diluted to 1 000 ml with water. - Standard acetate buffer: dissolve 40 g solid sodium hydroxide AR in 500 ml water in a beaker and allow to cool. Add 120 ml glacial acetic acid (3.2.7). Mix thoroughly, cool and transfer to a 1 000 ml volumetric flask. Make up to the mark with water. - Pyrrolidinedithiocarbamate solution (known as "carbate solution"): dissolve 103 mg sodium pyrrolidinedithiocarbamate (C5H8NNaS2.2H2O) in about 500 ml water, add 10 ml of n-amyl alcohol AR and 0.5 g NaHCO3 AR, and make up to 1 000 ml with water. - Copper sulphate solution (for standardization of 3.2.11). Stock solution Mix 1.249 g copper suphate (CuSO4.5H2O) AR with 50 ml 0.5M sulphuric acid and make up to 1 000 ml with water. Standard solution Mix 50 ml stock solution with 10 ml 0.5M H2SO4 and make up to 1 000 ml with water. - Sodium chloride AR - Gas-stripping apparatus (see Figure 5). The diameter of the sintered disc must be the same as the internal diameter of the cylinder. - Separating funnel, 250 ml. - Magnetic stirrer with magnet 25-30 mm. - Gooch crucible, diameter of the perforated base = 25 mm, type G 4. - Circular glass-fibre filter papers, 27 mm diameter with fibre diameter 0,5-1,5 m. - Two filter flasks with adaptors and rubber collars, 500 and 250 ml respectively. - Recording potentiometer fitted with a bright platinum indicator electrode and a calomel or silver/silver chloride reference electrode with a 250 mV range, with automatic burette of 20-25 ml capacity, or alternative manual equipment. - Method - Concentration and separation of the surfactant Filter the aqueous sample through a qualitative filter paper. Discard the first 100 ml of the filtrate. Into the stripping apparatus, previously rinsed with ethyl acetate, place a measured quantity of the sample, such that it contains between 250-800 mg non-ionic surfactant. To improve the separation add 100 g sodium chloride and 5 g sodium bicarbonate. If the volume of the sample exceeds 500 ml, add these salts to the stripping apparatus in solid form, and dissolve by passing nitrogen or air through. If a smaller-sized sample is used, dissolve the salts in 400 ml water and then add to the stripping apparatus. Add water to bring the level to the upper stopcock. Cautiously add 100 ml ethyl acetate on top of the water. Fill the wash-bottle in the gas-line (nitrogen or air) two-thirds full with ethyl acetate. Pass a gas stream of 30-60 l/h through the apparatus; the inclusion of a rotameter is recommended. The rate of aeration must be increased gradually at the beginning. The gas rate must be so adjusted that the phases remain noticeably separate to minimize the mixing of the phases and the solution of the ethyl acetate in the water. Stop the gas flow after five minutes. If there is a reduction of more than 20% in the volume of the organic phase through solution in water, the sublation must be repeated paying special attention to the rate of gas flow. Run off the organic phase into a separating funnel. Return any water in the separating funnel from the aqueous phase - it should only be a few ml - to the stripping apparatus. Filter the ethyl acetate phase through a dry qualitative filter paper into a 250 ml beaker. Put a further 100 ml ethyl acetate into the stripping apparatus and again pass nitrogen or air through for five minutes. Draw off the organic phase into the separating funnel used for the first separation, reject the aqueous phase and run the organic phase through the same filter as the first ethyl acetate portion. Rinse both the separating funnel and the filter with about 20 ml ethyl acetate. Evaporate the ethyl acetate extract to dryness on a water-bath (fume cupboard). Direct a gentle stream of air over the surface of the solution to accelerate the evaporation. - Precipitation and filtration Dissolve the dry residue from 3.3.1 in 5 ml methanol, add 40 ml water and 0.5 ml diluted HCl (3.2.3) and stir the mixture with a magnetic stirrer. To this solution add 30 ml of precipitating agent (3.2.6) from a measuring cylinder. The precipitate forms after repeated stirring. After stirring for 10 minutes leave the mixture to stand for at least five minutes. Filter the mixture through a Gooch crucible, the base of which is covered with a glass-fibre filter paper. First wash the filter under suction with about 2 ml glacial acetic acid. Then thoroughly wash the beaker, magnet, and crucible with glacial acetic acid, of which about 40-50 ml is necessary. It is not necessary to quantitatively transfer the precipitate adhering to the sides of the beaker, to the filter, because the solution of the precipitate for the titration is returned to the precipitating beaker, and the remaining precipitate will then be dissolved. - Solution of the precipitate Dissolve the precipitate in the filter crucible by the addition of hot ammonium tartrate solution (about 80°C, 353 K) (3.2.8) in three portions of 10 ml each. Allow each portion to stand in the crucible for some minutes before being sucked through the filter into the flask. Put the contents of the filter flask into the beaker used for the precipitation. Rinse the sides of the beaker with a further 20 ml of tartrate solution to dissolve the rest of the precipitate. Carefully wash the crucible, adaptor and filter flask with 150-200 ml water, and return the rinsing water to the beaker used for the precipitation. - The titration Stir the solution using a magnetic stirrer (3.2.16), add a few drops of bromocresol purple (3.2.5) and add the diluted ammonia solution (3.2.9) until the colour turns violet (the solution is weakly acid from the residue of acetic acid used for rinsing). Then add 10 ml standard acetate buffer (3.2.10), immerse the electrodes in the solution, and titrate potentiometrically with standard "carbate solution" (3.2.11), the burette tip being immersed in the solution. The titration rate should not exceed 2 ml/min. The endpoint is the intersection of the tangents to the two branches of the potential curve. It will be observed occasionally that the inflection in the potential curve becomes flattened; this can be eliminated by carefully cleaning the platinum electrode (by polishing with emery paper). - Blank determinations At the same time run a blank determination through the whole procedure with 5 ml methanol and 40 ml water, according to the instructions in 3.3.2. The blank titration should be below 1 ml, otherwise the purity of the reagents (3.2.3 - 3.2.7 - 3.2.8 - 3.2.9 - 3.2.10) is suspect, especially their content of heavy metals, and they must be replaced. The blank must be taken into account in the calculation of the results. - Control of the factor of the "carbate solution" Determine the factor for the carbate solution on the day of use. To do this, titrate 10 ml of the copper sulphate solution (3.2.12) with carbate solution after the addition of 100 ml water and 10 ml standard acetate buffer (3.2.10). If the amount used is "a" ml, the factor f is : f = 10/a and all the results of the titrations are multiplied by this factor. - Calculation of results Every non-ionic surfactant has its own factor, depending on its composition, particularly on the length of the alkene oxide chain. The concentration of non-ionic surfactant is expressed in relation to a standard substance - a nonyl phenol with 10 ethylene oxide units (NP 10) - for which the conversion factor is 0.054. Using this factor the amount of surfactant present in the sample is found expressed as mg of NP 10 equivalent, as follows: (b-c) . f . 0.054 = mg non-ionic surfactant as NP 10 where : b = volume of "carbate solution" used by the sample (ml), c = volume of "carbate solution" used by the blank (ml), f = factor of the "carbate solution". - Expression of results Express the results in mg/l as NP 10 to the nearest 0.1. FIGURES Figures 1, 2, 3, 4, 5 - see Official Journal No L 109, 22.4.1982, p.1 |